# Global Energy Transition: Model Description

# Version: GET-v12

April 2026

Fayas Malik Kanchiralla, Yi He, Maria de Oliveira Laurin, Selma Brynolf, Maria Grahn

Division of Maritime Environmental Sciences,

Department of Environment and Energy Sciences,

CHALMERS UNIVERSITY OF TECHNOLOGY

Gothenburg, Sweden

- Table of Contents

1.	Introduction	6

2.	Sets	7

2.1.	Time	7

2.2.	Regions	7

2.3.	Energy carriers and sources	8

2.4.	Conversion plant types	9

2.5.	Transport modes	9

2.6.	Engine / drivetrain types	10

3.	Scalars and Parameters	10

3.1.	Basic scalars	10

3.2.	Growth and depreciation parameters	11

3.3.	Technology lifetimes	11

3.4.	Energy conversion efficiency	12

3.5.	Fuel prices and supply potential	12

4.	Variables	12

4.1.	Primary energy and conversion	12

4.2.	Electricity sector	13

4.3.	Transport	13

4.4.	Capital	14

4.5.	Emissions and carbon	14

4.6.	Costs	14

5.	Equations	15

5.1.	Primary energy supply	15

5.1.1 Supply potential constraint	15

5.1.2 Fossil reserve constraint	15

5.1.3 Primary energy to conversion	15

5.2.	Import and export balances	15

5.2.1 Export equals sum of bilateral flows	15

5.2.2 Import equals sum of bilateral flows	16

5.2.3 Net trade supply balance	16

5.2.4 Second-loop supply cannot exceed production  [supply_loop_Q]	16

5.3.	Energy conversion and delivery	16

5.3.1 Secondary energy production	16

5.3.2 Electricity production	16

5.3.3 Co-generation secondary output	17

5.3.4 Energy delivery	17

5.3.5 Final demand balance (non-electricity, non-solid heat)	17

5.3.6 Heat demand with DAC	17

5.3.7 Nuclear production growth limit	17

5.4.	Sliced electricity sector	18

5.4.1 Slice electricity demand	18

5.4.2 Sector electricity use per slice	18

5.4.3 Sliced electricity balance	18

5.4.4 Total storage losses	19

5.4.5 Annual generation equals annual conversion	19

5.4.6 Thermal ramp constraint	19

5.4.7 Capital limits for dispatchable and storage technologies	19

5.4.8 Resource class capital limits and aggregation	20

5.5.	Sliced hydrogen sector	20

5.5.1 Annual H₂ production equals annual conversion	20

5.6.	Transport module	21

5.6.1 Transport fuel balance	21

5.6.2 Mode energy-service demand balance	21

5.6.3 Non-PHEV, non-FCREEV vehicle capacity constraint	21

5.6.4 PHEV liquid-fuel consumption	22

5.6.5 PHEV electricity-fuel split	22

5.6.6 FCREEV vehicle constraints	22

5.6.7 Electric vehicle fleet limit	22

5.6.8 Pilot fuel requirement	22

5.7.	Investments and depreciation	23

5.7.1 Initial capital stock	23

5.7.2 Capital stock evolution	23

5.7.3 Engine stock evolution	23

5.7.4 Infrastructure stock evolution	23

5.7.5 Capital limits on conversion	23

5.7.6 Infrastructure capacity limit	24

5.8.	Emission calculations	24

5.8.1 Upstream emissions	24

5.8.2 Shipping emissions by vessel type	24

5.8.3 Total ship emissions	24

5.8.4 Road and aviation transport emissions	25

5.8.5 Total regional CO₂ emissions	25

5.8.6 Global emissions	25

5.8.7 Aggregated cumulative emissions	25

5.8.8 Carbon capture	25

5.8.9 Carbon balance and permanent storage	25

5.9.	Technology restrictions	26

5.9.1 Co-generation limit	26

5.9.2 Solar heat limits	26

5.9.3 Heat pump limits	26

5.9.4 Biomass chip limit in solid heat	26

5.9.5 Feedstock limits	26

5.10.	Expansion rate constraints	27

5.10.1 Primary supply growth	27

5.10.2 Infrastructure growth	27

5.10.3 Engine fleet growth	27

5.10.4 Investment growth in conversion plants	27

5.10.5 Coal phase-out protection	28

5.10.6 Biomass phase-out protection	28

5.10.7 DAC growth limit	28

5.11.	Cost accounting	28

5.11.1 Primary energy extraction and import cost	28

5.11.2 Shipping levy cost	28

5.11.3 Capital investment cost	28

5.11.4 O&M cost	29

5.11.5 Distribution cost	29

5.11.6 Carbon storage cost	29

5.11.7 Direct air capture cost	29

5.11.8 Carbon tax (optional)	30

5.11.9 Total annual cost	30

5.11.10 Objective function: total discounted cost	30

6.	Constraints and Scenario Settings	30

6.1.	CO₂ cumulative budget	30

6.2.	Technology availability	30

6.3.	Calibration constraints	31

7.	Solver Settings and Implementation	31

# Introduction

The Global Energy Transition model version 12.0 (GET v12) is a linear-programming, cost-minimising global energy systems model coded in GAMS and solved with CPLEX. It builds on more than two decades of development at Chalmers University of Technology (Azar et al. 2000). The model identifies the least-cost mix of primary energy supply, conversion technologies, transport fuels and vehicles, and negative-emission options that meets exogenously prescribed final-energy demand while respecting a cumulative CO₂ budget. The aim of the report is to describe the algorithms and modelling framework of GET v12 to the users, reviewers, and collaborators that helps in better understanding of model and result analysis.

It is important to remember that energy systems cost-minimizing models do not predict the future and are not designed to forecast the future development of the energy system. Instead, they provide a tool to understand system behavior, interactions and connections among energy technology options in different sectors.

In GET model, the world is treated as ten distinct regions with unimpeded movement of energy resources between regions. The model time horizon runs from 2010 to 2150 in 10-year steps. All monetary values are in real USD 2010. The discount rate is 5% per year. The GET v12 comprises ten modules (Figure 1): primary energy supply; energy conversion and storage; carbon capture; energy export and import, energy distribution; carbon budget and emission conversion; the electricity sector (incorporating time slices for regional variable renewable energy conditions); the transport sector; industrial feedstocks; and the heat sector. In the model, primary energy sources (crude oil, natural gas, coal, nuclear, hydro, biomass, wind, and solar) are converted into various energy carriers to satisfy the energy demands of end-use sectors. The technical and economic relationships between technologies are parameterized using efficiencies, load factors, costs, and lifetimes. The model includes several constraints such as annual or total supply limits on available energy sources; expansion constraints for investments; load balance equilibrium; carbon dioxide (CO2) budgets; limitation on export and import (e.g., electricity); and permanent carbon-storage capacity availability.

This report follows the same structure as the GET-RC 6.1 description. Section 2 describes the sets; Section 3 the parameters and scalars; Section 4 the variables; Section 5 all equations with detailed explanations; Section 6 the constraints and scenario settings; Section 7 the carbon cycle module; and Section 8 solver settings.

Figure 1: Different modules of the GETv12

# Sets

Sets are the indices of the model. They are defined in sets.gms. GAMS sets are case-insensitive. In this report, set names are written in italic when they appear in equation descriptions.

## Time

The full time axis t_a runs from 1800 to 2150 in 10-year steps. The model period t (subset of t_a) is 2010–2150. Historical steps t_h (1800–2000) are used only in the carbon-cycle equations. The first model year is init_year = {2010}, which is used to set initial capacity conditions. The extended horizon t_ext = {2110, 2120, 2130, 2140, 2150} covers post-2100 dynamics.

| Set | Contents | Purpose |
| --- | --- | --- |
| t_a | 1800, 1810, 1820, 1830, 1840, 1850, 1860, 1870, 1880, 1890, 1900, 1910, 1920, 1930, 1940, 1950, 1960, 1970, 1980, 1990, 2000, 2010, 2020, 2030, 2040, 2050, 2060, 2070, 2080, 2090, 2100, 2110, 2120, 2130, 2140, 2150 | Full time axis |
| t | 2010, 2020, 2030, 2040, 2050, 2060, 2070, 2080, 2090, 2100, 2110, 2120, 2130, 2140, 2150 | Active optimisation period |
| t_h | 1800, 1810, 1820, 1830, 1840, 1850, 1860, 1870, 1880, 1890, 1900, 1910, 1920, 1930, 1940, 1950, 1960, 1970, 1980, 1990, 2000 | Historical steps for carbon cycle calibration |
| t_1 | 2010, 2020, 2030, 2040, 2050, 2060, 2070, 2080, 2090, 2100, 2110, 2120, 2130 | Active steps excluding last two (used in growth constraints) |
| init_year | 2010 | Base year for initial capital and engine stocks |
| end_year | 2100 | end of main scenario period |

## Regions

Ten world regions (reg) follow the MESSAGE/IIASA regionalisation. The sub-groups OECD, MIC (middle-income countries) and ROW (rest of world) are used when parameterising region-specific technology diffusion rates.

| Code | Region | Sub-group |
| --- | --- | --- |
| EUR | Europe | OECD |
| NAM | North America | OECD |
| PAO | Pacific OECD (Aus, Japan, NZ) | OECD |
| FSU | Former Soviet Union | MIC |
| LAM | Latin America & Caribbean | MIC |
| CPA | Centrally Planned Asia (mainly China) | MIC |
| AFR | Sub-Saharan Africa | ROW |
| SAS | South Asia (mainly India) | ROW |
| PAS | Other Pacific Asia | ROW |
| MEA | Middle East & North Africa | ROW |

The sets reg_imp and reg_exp are both identical to reg and are used to form bilateral trade pairs (import_from) in the energy trade equations.

## Energy carriers and sources

The master set energy contains all primary and secondary energy carriers tracked in the model. Key subsets are:

| Subset | Role | Selected members |
| --- | --- | --- |
| Energy | All energy resource | oil1,oil2,oil3, bio, bio1, bio2, bio3,hydro, wind, solar, gas1, gas2, gas3, oil, coal1,coal2, coal, MeOH, H2, elec, petro, air_fuel,trsp, pellets, CH4 , central_heat, dist_heat, solid_heat, non_solid_heat, feed-stock, uranium1, uranium2, uranium3, uranium4, uranium5, biogas, ethanol, biodiesel, PuR, EU4, EU1, Pu,FBF, MOX, Rep1, Rep2, Rep3, sLWRf, BrProd, sCANDUf, sMoxf, waste, CANDUw, MOXw, Brwaste, windonshoreA, windonshoreB, windoffshore, pvrooftop, pvplantA, pvplantB, cspplantA, cspplantB, storage_12h, storage_24h, storage_48h, storage_96h, air, CO2, NH3, LCH4, LH2, CH2,agrifeedstock |
| primary(en_in) | Primary resources extracted from nature | bio1–3, coal1–2, oil1–3, gas1–3, uranium1–5, wind, solar, hydro |
| second_in(energy) | Secondary carriers that can be traded or re-converted | bio, oil, elec, MeOH, biogas, ethanol, biodiesel, petro, air_fuel, coal, pellets, EU4, EU1, Pu, FBF, MOX, Rep1, Rep2, Rep3, sLWRf, BrProd, sCANDUf, sMoxf, NH3, CH4, LCH4, LH2, CH2,H2 |
| en_in(energy) | All inputs to en_conv (primary + secondary + storage) | oil2, oil1,oil3, coal1, coal2, bio1, bio2, bio3, bio, hydro, wind, solar, gas1, gas2, gas3, oil, coal, MeOH, biogas, ethanol, biodiesel, H2, elec, petro, air_fuel, CH4, pellets, uranium1, uranium2, uranium3, uranium4, uranium5, PuR, EU4, EU1, Pu, FBF, MOX, Rep1, Rep2, Rep3, sLWRf, BrProd, sCANDUf, sMoxf, windonshoreA, windonshoreB, windoffshore, pvrooftop, pvplantA, pvplantB, cspplantA, cspplantB, storage_12h, storage_24h, storage_48h, storage_96h, air, NH3, LCH4, LH2, CH2 |
| en_out(energy) | All outputs from en_conv | oil, elec, MeOH, biogas, ethanol, biodiesel, H2, petro, air_fuel ,trsp, CH4, coal, central_heat, dist_heat, solid_heat, non_solid_heat, feed-stock,agrifeedstock, pellets, bio, EU4, EU1, Pu, FBF, MOX, Rep1, Rep2, Rep3, sLWRf, BrProd, sCANDUf, sMoxf, waste, CANDUw, MOXw, PuR, Brwaste, NH3, LCH4, LH2, CH2, CO2 |
| sector(energy) | Final demand sectors (must be satisfied) | elec, central_heat, dist_heat, solid_heat, non_solid_heat, feed-stock ,h2, MeOH, biogas, ethanol, biodiesel,oil, NH3, LCH4, LH2, CH2,agrifeedstock |
| trsp_fuel(energy) | Transport fuels | elec, MeOH, biogas, ethanol, biodiesel, petro, NH3, LCH4, LH2, CH2, CH4 |
| storage_tech(energy) | Electricity storage | storage_12h, storage_24h, storage_48h, storage_96h |
| sliced_H2(en_in) | H₂ production routes with time slices | bio, biogas, CH4, coal, elec, oil, petro, solar |
| class_tech(en_in) | Variable renewables with resource classes | windonshoreA/B, windoffshore, pvrooftop, pvplantA/B, cspplantA/B |
| ccs_fuel(energy) | Fuels from which carbon can be captured | coal, oil, CH4, bio, biogas, pellets |
| nuclear_fuel | All nuclear materials | uranium1–5, EU4, EU1, Pu, FBF, MOX, Rep1–3, sLWRf, BrProd… |

## Conversion plant types

The set type defines how a conversion plant operates. Every en_conv variable is indexed by one of these four types. The subset C_capt contains the two types that apply carbon capture (dec and cg_dec). The subset CG_type contains the two co-generation types (cg and cg_dec).

| Type | Description |
| --- | --- |
| 0 | Conventional plant – no co-generation, no CCS |
| cg | Co-generation of electricity and heat, no CCS |
| dec | Decarbonised plant with carbon capture and storage (CCS) |
| cg_dec | Co-generation plant with CCS |

## Transport modes

The set trsp_mode contains 21 transport modes. trsp_mode_no_train excludes the two rail modes (p_rail, f_rail) from investment and engine equations since rail is assumed to be electrified by infrastructure not tracked in the vehicle engine model.

| Subset | Members |
| --- | --- |
| car_or_truck | p_car, f_LCV, f_MDT_LH, f_MDT_RD, f_HDT_LH, f_HDT_RD, p_bus |
| heavy_mode (shipping) | f_container, f_cargo_bulk, f_oiltanker, f_gastanker, f_cargo_short, p_ferry_short, p_ferry_long, f_othership |
| air_mode | p_air_short, p_air_medium, p_air_long, f_air |
| truck | f_LCV, f_MDT_LH, f_MDT_RD, f_HDT_LH, f_HDT_RD |
| Engine4S (4-stroke marine engines) | f_cargo_short, p_ferry_short, p_ferry_long, f_othership |
| Engine2S (2-stroke marine engines) | f_container, f_cargo_bulk, f_oiltanker, f_gastanker |

## Engine / drivetrain types

The set engine_type contains six drivetrains. The subset elec_veh contains those that use electricity (BEV, PHEV, FCREEV). The subset non_phev is used in the standard vehicle capacity equation (all drivetrains except PHEV, which has its own equation). IC_FC covers the two pure combustion/fuel-cell types.

| engine_type | Description |
| --- | --- |
| 0 | Internal combustion engine (ICE) – runs on liquid or gaseous fuels |
| FC | Fuel cell vehicle – fuel converted to electricity on board |
| hyb | Conventional hybrid – regenerative braking, no plug-in |
| BEV | Battery electric vehicle – charged from grid only |
| PHEV | Plug-in hybrid – uses both liquid fuel and grid electricity |
| FCREEV | Fuel cell range-extended EV – FC as range extender, BEV primary drive |

# Scalars and Parameters

In GAMS terminology, parameters are given (exogenous) data. Scalars are single-value parameters. Parameters are multi-dimensional. In this report, parameter names are printed in blue italic when they appear in equations; variable names are in red.

## Basic scalars

| Scalar | Value | Unit | Description |
| --- | --- | --- | --- |
| Msec_per_year | 31.6 | Ms/yr | Seconds per year in millions – used to convert TW capacity to EJ/yr |
| t_step | 10 | yr | Length of each time step |
| r | 0.05 | – | Annual discount rate for the objective function |
| r_invest | 0.05 | – | Annual interest rate applied to investments (set equal to r in base case) |
| cost_strg | 0.010 | GUSD/MtCO₂ | Unit cost of permanent CO₂ geological storage |
| CO2aircapt_cost | 0.5 | GUSD/MtCO₂ | Unit cost of direct air capture |
| heat_CO2aircapture | 0.0047 | EJ/MtCO₂ | Thermal energy demand per unit CO₂ captured by DAC |
| El_DAC | 0.00252 | EJ/MtCO₂ | Electricity demand per unit CO₂ captured by DAC |
| WTT_CH4 | 6.24 | MtCO₂eq/EJ | Well-to-tank methane leakage (1 % leakage assumed) |
| Distrib_LCH4 | 3.12 | MtCO₂eq/EJ | LCH4 distribution leakage (0.5 % leakage) |
| TTW_N2O | 4.023 | MtCO₂eq/EJ | Tank-to-wheel N₂O from NH3 combustion (0.03 g/kWh) |
| cogen_fr_h | 0.6 | – | Maximum fraction of central heat demand from co-generation plants |
| max_chip | 0.67 | – | Maximum fraction of solid heat demand met by wood chips |

## Growth and depreciation parameters

To prevent unrealistically fast transitions, the model imposes both relative and absolute bounds on how quickly investment, supply, infrastructure and engine fleets can grow from one decade to the next. All growth constraints include a small kick-start value (init_*) so that new technologies can enter even when the current stock is zero. The relative and absolute growth limits are:

| Parameter | Value | Unit | Description |
| --- | --- | --- | --- |
| cap_g_lim | 0.25 | /decade | Max relative growth of annual investment in conversion plants |
| init_plant | 0.0002 | TW/decade | Kick-start for new conversion plant investment |
| supply_lim | 0.10 | /decade | Max relative growth of primary energy extraction |
| init_supply | 4 | EJ/decade | Kick-start for primary energy supply |
| infra_g_lim | 0.10 | /decade | Max relative growth of transport infrastructure capacity |
| init_infra | 0.01 | TW/decade | Kick-start for new infrastructure |
| eng_g_lim | 0.20 | /decade | Max relative growth of each engine-type fleet |
| init_eng | 0.01 | Gveh/decade | Kick-start for new engine fleet |

## Technology lifetimes

The default plant lifetime is 25 yrs. The following technologies have different lifetimes:

| Technology | Lifetime (yr) |
| --- | --- |
| Hydro power plants | 40 |
| Nuclear reactors (EU4, EU1, MOX, FBF) | 40 |
| EU4 hydrogen plants (cg type) | 40 |
| Road vehicle engines (car_or_truck) | 15 |
| All other transport engines (ships, aircraft) | 30 |
| All fuel infrastructure | 50 |

## Energy conversion efficiency

The parameter effic(en_in, en_out, type, reg, t) is the time- and region-dependent conversion efficiency. It is computed from effic_0 (the mature/ideal efficiency) and the current near-term efficiency, using a linear interpolation over a technology development time t_tech_effic = 40 years. The diffusion rate is region-group dependent: OECD regions start at 90 % of effic_0, MIC regions at 80 %, and ROW regions at 70 %. The formula used (after params.gms) is:

effic(en_in, en_out, type, reg, t) = min( effic_0,

(effic_0 - effic_current * group_factor) / t_tech_effic

* (ord(t)-1) * t_step + effic_current * group_factor )

where group_factor is 0.7 for ROW, 0.8 for MIC, and 0.9 for OECD. Nuclear fuel efficiencies are held fixed at effic_0 for all regions.

## Fuel prices and supply potential

Primary fuel prices price(fuels, reg) are in USD2010/GJ (equivalent to GUSD/EJ). Three resource grades exist for fossil fuels and biomass; the model uses the cheapest grade first and then progresses to higher grades as lower grades are exhausted. The cumulative supply potentials supply_pot_0(primary, reg) are in EJ.

The shipping emission factors emis_fact2(trsp_fuel) are the tank-to-wake CO₂ intensities used in shipping and aviation emission calculations. Upstream emission factors emis_fact(energy) are in MtCO₂/EJ and are used in the upstream emission equation.

# Variables

Variables are the decision variables optimised by the LP solver. In GET 12, all variables except tot_cost are positive (declared as positive variables in declare.gms). The variable tot_cost is free (can take any sign). In this report, variable names are shown in bold.

## Primary energy and conversion

| Variable | Indices | Unit | Description |
| --- | --- | --- | --- |
| supply_1 | primary, reg, t | EJ/yr | Primary energy extracted per region per period |
| en_conv | energy, en_out, type, reg, t | EJ/yr | Energy converted from en_in to en_out (primary energy units, i.e. before efficiency losses) |
| supply_2 | second_in, reg, t | EJ/yr | Secondary carrier fed back into conversion (H2, electricity, etc.) |
| energy_prod | en_out, reg, t | EJ/yr | Energy produced in secondary/output units (after efficiency losses), before trade |
| energy_prod2 | en_in, en_out, type, reg, t | EJ/yr | Technology-specific energy production (diagnostic variable) |
| energy_deliv | en_out, reg, t | EJ/yr | Energy delivered to final demand after trade and second-loop conversion |
| supply_sec | energy, reg, t | EJ/yr | Co-generation secondary output from all conversion processes |
| export / import | second_in, reg, t | EJ/yr | Aggregate export and import per carrier per region |
| import_from | second_in, reg_imp, reg_exp, t | EJ/yr | Bilateral trade flow from reg_exp to reg_imp |

## Electricity sector

| Variable | Description |
| --- | --- |
| elec_deliv(reg, slice, t) | Electricity delivered to all users in each time slice (EJ per slice) |
| elec_gen(en_in, type, reg, slice, t) | Electricity generated by technology (en_in, type) in each slice (EJ) |
| elec_use(elecsector, reg, slice, t) | Electricity consumed by end-use sector in each slice (EJ) |
| storage_slice_transfer(tech, reg, s1, s2, t) | Energy transferred from slice s1 to s2 via storage technology (EJ) |
| elec_losses(reg, t) | Total annual electricity losses from storage (EJ/yr) |
| capital_class(class_tech, class, reg, t) | Installed capacity per wind/solar resource class (TW) |
| elec_gen_class(class_tech, class, reg, slice, t) | Generation per resource class and time slice (EJ) |
| elec_decarb(reg, t) | Extra electricity demand from CCS decarbonisation plants (EJ/yr) |

## Transport

| Variable | Description |
| --- | --- |
| trsp_energy(trsp_fuel, engine_type, trsp_mode, reg, t) | Energy used per fuel-drivetrain-mode combination (EJ/yr) |
| engines(trsp_fuel, engine_type, trsp_mode, reg, t) | Aggregated vehicle/engine stock (Gvehicles for road, and Mvehicles for shipping and aviation) |
| eng_invest(trsp_fuel, engine_type, trsp_mode, reg, t) | New engine investment per decade (Gvehicles/decade) |
| infra(trsp_fuel, reg, t) | Transport fuel infrastructure capacity (TW) |
| infra_invest(trsp_fuel, trsp_mode, reg, t) | New infrastructure investment (TW/decade) |

## Capital

| Variable | Description |
| --- | --- |
| capital(energy, en_out, type, reg, t) | Aggregated capital stock in conversion plants (TW) |
| cap_invest(energy, en_out, type, reg, t) | New investment in conversion capacity per decade (TW/decade) |

## Emissions and carbon

| Variable | Unit | Description |
| --- | --- | --- |
| C_emission(reg, t) | MtCO₂ | Annual CO₂-equivalent emissions per region |
| Tot_emis(t) | MtCO₂ | Global annual emissions |
| agg_emis | MtCO₂ | Cumulative global emissions over model horizon |
| transp_emis(reg, t) | MtCO₂ | Road and non-heavy transport emissions |
| ship_emis(reg, t) | MtCO₂ | Total shipping emissions per region |
| ship_emis2(heavy_mode, reg, t) | MtCO₂ | Shipping emissions by vessel type |
| emiss_upstream(reg, t) | MtCO₂ | Upstream (non-transport) emissions including methane leakage |
| C_capt_tot(reg, t) | MtCO₂ | Total CO₂ captured (BECCS + fossil CCS + DAC) |
| C_strg_tot(reg, t) | MtCO₂ | CO₂ permanently stored per region per period |
| C_strg_agg | MtCO₂ | Cumulative permanent storage over model horizon |
| C_capt_air(reg, t) | MtCO₂ | CO₂ removed by direct air capture |
| Pu_stock(reg, t) | EJ | Plutonium inventory available for fast-breeder reactors |

## Costs

| Variable | Unit | Description |
| --- | --- | --- |
| cost_fuel(reg, t) | GUSD | Primary energy extraction + import cost |
| cost_cap(reg, t) | GUSD | Capital cost: plants, engines, infrastructure |
| OM_cost(t) | GUSD | Operation and maintenance cost (global, all regions) |
| dist_cost(t) | GUSD | Energy distribution cost (heat networks + fuel infrastructure) |
| cost_C_strg(t) | GUSD | Carbon transport and geological storage cost |
| cost_capt_air(t) | GUSD | Direct air capture cost |
| tax(reg, t) | GUSD | Carbon tax payment |
| ship_levy_cost(t) | GUSD | Revenue from shipping emission levy |
| annual_cost(t) | GUSD | Total global energy system cost in period t |
| tot_cost | GUSD | NPV of total energy system cost – the objective function |

# Equations

In this section all equations are presented with mathematical notation followed by explanatory prose in the style of the GET-RC 6.1 description. Equation numbers are given in parentheses at the right margin. The GAMS equation name is given in the subheading for cross-reference with the code.

## Primary energy supply

### 5.1.1 Supply potential constraint

The annual extraction of each primary energy source in each region cannot exceed the exogenously given supply potential. For renewable sources (wind, solar, hydro) supply_pot is an annual flow limit (EJ/yr); for fossil fuels supply_pot is set equal to supply_pot_0 (total reserves in EJ) so the cumulative reserve constraint (Eq. 2) governs instead.

supply_pot(primary, reg, t)  >=  supply_1(primary, reg, t)	(1)

### 5.1.2 Fossil reserve constraint

The cumulative extraction of each fossil-fuel grade in each region over the entire model horizon cannot exceed the total recoverable resource endowment supply_pot_0. Because the time step is 10 years, each decade's supply must be multiplied by t_step before summing:

SUM_t [ t_step * supply_1(stock, reg, t) ]  <=  supply_pot_0(stock, reg)	(2)

### 5.1.3 Primary energy to conversion

All primary energy extracted in a region must enter the energy conversion module en_conv. The equation sums across all plant types and all produced energy carriers to ensure that nothing is extracted and then discarded:

supply_1(primary, reg, t)  =  SUM_{type, en_out} en_conv(primary, en_out, type, reg, t)	(3)

## Import and export balances

### 5.2.1 Export equals sum of bilateral flows

The total export of an energy carrier from a region to all destinations equals the sum of bilateral flows import_from from that region. This equation ensures that every unit exported is accounted for by a corresponding bilateral flow:

export(second_in, reg_exp, t)  =  SUM_{reg_imp} import_from(second_in, reg_imp, reg_exp, t)	(4)

### 5.2.2 Import equals sum of bilateral flows

Symmetrically, the total import to a region equals the sum of all bilateral flows arriving from other regions:

import(second_in, reg_imp, t)  =  SUM_{reg_exp} import_from(second_in, reg_imp, reg_exp, t)	(5)

### 5.2.3 Net trade supply balance

For each secondary carrier in each region, the net supply after trade (production minus exports plus imports) must equal the amount fed into the conversion module as a second-stage input. This equation links the trade variables with the conversion variable for carriers such as H2, MeOH, coal, and LCH4:

supply_2(second_in,reg,t) - export(second_in,reg,t) + import(second_in,reg,t)	  =  SUM_{type,en_out} en_conv(second_in, en_out, type, reg, t)	(6)

### 5.2.4 Second-loop supply cannot exceed production  [supply_loop_Q]

The amount of a secondary carrier returned for second-stage conversion cannot exceed the total amount produced of that carrier. This prevents circular flows where more energy is recycled than was ever produced:

supply_2(second_in, reg, t)  <=  energy_prod(second_in, reg, t)	(7)

## Energy conversion and delivery

### 5.3.1 Secondary energy production

For all output energy carriers except electricity and hydrogen (which have dedicated sliced-sector equations), the total production equals the sum of conversions across all input fuels and plant types, weighted by the conversion efficiency. The term supply_sec captures co-generated secondary outputs (heat from CHP plants) that are added to the main output stream:

energy_prod(en_out, reg, t)  =  SUM_{type, en_in} en_conv(en_in, en_out, type, reg, t)	  * effic(en_in, en_out, type, reg, t)  +  supply_sec(en_out, reg, t)	(8)

### 5.3.2 Electricity production

For electricity, wind and solar aggregate variables are excluded from the en_conv sum (they are represented through the individual sub-technologies in the sliced sector to avoid double-counting). Storage losses are subtracted. The result is annual electricity production in secondary energy terms (EJ/yr):

energy_prod('elec', reg, t)  =  SUM_{type,en_in [excl. solar,wind]} en_conv * effic +  supply_sec('elec', reg, t)  -  elec_losses(reg, t)	(9)

### 5.3.3 Co-generation secondary output

The parameter heat_effic(en_in, type, en_out, energy) gives the fraction of the primary energy throughput in en_conv that appears as a secondary output (e.g. distributed heat from a gas-fired CHP plant). Summing over all conversion processes gives the total secondary output for each carrier:

supply_sec(energy, reg, t)  =  SUM_{en_in, type, en_out} en_conv(en_in,en_out,type,reg,t)* heat_effic(en_in, type, en_out, energy)	(10)

### 5.3.4 Energy delivery

The energy available for final demand (energy_deliv) equals production minus the amount recycled back into conversion. The subtraction of supply_2 removes from the delivery balance those secondary carriers that are being used as inputs to another conversion stage:

energy_deliv(en_out, reg, t)  =  energy_prod(en_out, reg, t)  -  supply_2(en_out, reg, t)	(11)

### 5.3.5 Final demand balance (non-electricity, non-solid heat)

For each demand sector other than electricity and solid heat, the energy delivered must exactly equal the exogenously given demand dem(sector, reg, t). This makes the demand balance an equality rather than an inequality, forcing the model to supply exactly what is needed:

energy_deliv(sector, reg, t)  =  dem(sector, reg, t)     [for sector not in {elec, solid_heat}]	(12)

### 5.3.6 Heat demand with DAC

When direct air capture is deployed, it requires heat. The solid-heat demand balance therefore augments the exogenous demand by the heat consumed by DAC (the parameter heat_CO2aircapture converts MtCO₂ captured into EJ of heat required):

energy_deliv('solid_heat', reg, t)  =  dem('solid_heat', reg, t)

+  C_capt_air(reg, t) * heat_CO2aircapture	(13)

### 5.3.7 Nuclear production growth limit

To prevent unrealistically rapid nuclear expansion, the energy produced from any nuclear carrier in one period cannot exceed the previous period's production by more than 0.025 EJ – the annual output of a single 1-GW reactor (1 GW × 0.8 load factor × 8760 h × t_step = 0.025 EJ per decade per region):

energy_prod(nuclear_just, reg, t+1)  <=  energy_prod(nuclear_just, reg, t)  +  0.025	(14)

## Sliced electricity sector

The electricity sector is modelled across numclusters = 16 representative time slices per year, derived from k-means clustering of hourly demand and renewable availability data. Each slice s has a weight slice_lng(reg, s) that sums to unity over the year. The sliced approach captures the temporal mismatch between variable renewable supply and demand without requiring a full chronological resolution.

### 5.4.1 Slice electricity demand

The electricity delivered in each slice must equal the slice demand. The slice demand is distributed from the annual total using the slice weight and sliced_demand profile. DAC electricity demand El_DAC is added as a uniform load:

elec_deliv(reg, slice, t)  =  energy_deliv('elec', reg, t) * slice_lng(reg,slice)* sliced_demand(reg, slice)  	(15)

### 5.4.2 Sector electricity use per slice

The electricity used by each end-use sector (such as central heating or road transport) in a given slice is proportional to the annual conversion en_conv('elec', elecsector, '0', reg, t) scaled by the slice weight:

elec_use(elecsector, reg, slice, t)  =  en_conv('elec', elecsector, '0', reg, t)  * slice_lng(reg, slice) * sliced_demand(reg, slice)	(16)

### 5.4.3 Sliced electricity balance

This is the core electricity balance in each time slice. Generation (from all dispatchable and variable plants, excluding the aggregate wind and solar variables to avoid double-counting) must equal slice demand, plus end-use sector load, minus storage discharge (which returns electricity to the system with efficiency losses), plus storage charging, plus the electricity consumed by hydrogen electrolysis:

SUM_{en_in,type [excl. solar,wind aggregate]} elec_gen(en_in,type,reg,slice,t)

=  elec_dem(reg,slice,t)+ SUM_{elecsector} elec_use(elecsector,reg,slice,t) -  SUM_{storage,s2} storage_slice_transfer(storage,reg,s2,slice,t) *effic_storage + SUM_{storage,s2}storage_slice_transfer(storage,reg,slice,s2,t) + H2_prod('elec','0',reg,slice,t) / effic('elec','H2','0',reg,t) +  C_capt_air(reg, t) * El_DAC		(17)

The first storage term represents discharging (energy coming back from storage, multiplied by round-trip efficiency). The second term represents charging (energy going into storage). Electrolysis adds to the electricity load.

### 5.4.4 Total storage losses

Annual electricity losses from storage are accumulated across all storage types, slices and slice-pairs. The loss per unit of discharge is (1 – efficiency):

elec_losses(reg, t)= SUM_{storage,s1,s2}storage_slice_transfer(storage,reg,s2,s1,t) * 1  - effic(storage,'elec','0',reg,t))	(18)

### 5.4.5 Annual generation equals annual conversion

The sum of slice-level generation for a technology over the year must equal the annual energy conversion multiplied by efficiency. This links the sliced sub-model to the aggregate en_conv variable used elsewhere in the model:

SUM_slice elec_gen(en_in, type, reg, slice, t)= en_conv(en_in,'elec',type,reg,t)* effic(en_in,'elec',type,reg,t)	(19)

### 5.4.6 Thermal ramp constraint

Dispatchable plants (coal, gas, biomass, nuclear, hydro) cannot produce at full capacity in one slice and zero in another, since ramping takes time. The constraint limits the ratio of generation in two different slices relative to their lengths. The equation is rearranged to avoid division by zero for zero-length slices:

elec_gen(thermalelec,type,reg,slice,t) * slice_lng(reg,slice2)>=  ramp_lim(thermalelec,type) * elec_gen(thermalelec,type,reg,slice2,t)* slice_lng(reg,slice)	(20)

The parameter ramp_lim(thermalelec, type) is the minimum output ratio. A ramp_lim of 0.5 means output in any slice cannot be less than half the output of any other slice (normalised by slice length).

### 5.4.7 Capital limits for dispatchable and storage technologies

For non-class technologies, generation in each slice is limited by installed capacity multiplied by the slice-specific load factor lfs and the number of seconds in the slice:

elec_gen(en_in,type,reg,slice,t)  <=  capital(en_in,'elec',type,reg,t)

* lfs(en_in,type,reg,slice,'elec') * Msec_per_year * slice_lng(reg,slice)	(21)

Storage transfers between slices are limited by installed capacity multiplied by the hours per year available for the particular slice-to-slice transfer (storage_slice_transfer_times, in hours/year). Capital is in TW, energy in EJ, hence the 3600/1e6 conversion factor:

storage_slice_transfer(tech,reg,s1,s2,t)  <=  capital(tech,'elec','0',reg,t)

* storage_slice_transfer_times(tech,reg,s1,s2) * 3600 / 1e6	(22)

### 5.4.8 Resource class capital limits and aggregation

Wind and solar sub-technologies (onshore A/B, offshore, PV rooftop/plantA/B, CSP A/B) are organised into resource classes c1–c5 with class-specific load factor profiles lfsl(tech, class, reg, slice) and upper capacity limits class_capacity_limit. Generation per class per slice is:

elec_gen_class(class_tech,class,reg,slice,t)<= capital_class(class_tech,class,reg,t)* lfsl(class_tech,class,reg,slice) * Msec_per_year * slice_lng(reg,slice)	(23)

The sum over classes equals the total technology generation:

SUM_class elec_gen_class(class_tech,class,reg,slice,t)  =  elec_gen(class_tech,'0',reg,slice,t)	(24)

SUM_class capital_class(class_tech,class,reg,t)  =  capital(class_tech,'elec','0',reg,t)	(25)

The wind and solar aggregate variables (wind, solar) are sums of the sub-technology variables:

elec_gen('wind','0',reg,slice,t)  =  gen(windonshoreA) + gen(windonshoreB) + gen(windoffshore)	(26)

elec_gen('solar','0',reg,slice,t) =  gen(pvrooftop) + gen(pvplantA) + gen(pvplantB)	(27)

elec_gen('solar','cg',reg,slice,t)=  gen(cspplantA) + gen(cspplantB)	(28)

## Sliced hydrogen sector

### 5.5.1 Annual H₂ production equals annual conversion

Exactly as for the electricity sector, the sum of slice-level H₂ production must equal the annual en_conv times efficiency. This links the sliced H₂ sub-model with the aggregate conversion variable:

SUM_slice H2_prod(sliced_H2, type, reg, slice, t)  =  en_conv(sliced_H2,'H2',type,reg,t)	(29)

* effic(sliced_H2,'H2',type,reg,t)

## Transport module

The transport module links the macro-level energy conversion variable en_conv with mode-specific fuel and vehicle technology choices. The chain is: primary energy → en_conv(trsp_fuel, 'trsp', type) → trsp_energy(trsp_fuel, engine_type, trsp_mode) → trsp_demand(trsp_mode). The conversion factor trsp_conv(trsp_fuel, engine_type, trsp_mode) converts energy into useful transport service (tkm or pkm) and reflects drivetrain efficiency differences.

### 5.6.1 Transport fuel balance

The total fuel of each type used across all modes and drivetrains must equal the fuel delivered from the energy conversion module. This connects the disaggregated transport variables to the rest of the energy system:

SUM_{trsp_mode, engine_type} trsp_energy(trsp_fuel, engine_type, trsp_mode, reg, t)  =  SUM_{type} en_conv(trsp_fuel,'trsp',type,reg,t) * effic(trsp_fuel,'trsp',type,reg,t) 	(30)

### 5.6.2 Mode energy-service demand balance

For each transport mode the model must meet the exogenous demand for transport services (trsp_demand, in EJ of useful service). The fuel energy is multiplied by the conversion factor trsp_conv to translate energy into service. Different drivetrains require different amounts of energy for the same service (e.g. a fuel cell vehicle is more efficient than an ICE):

trsp_demand(trsp_mode, reg, t)  =  SUM_{trsp_fuel, engine_type}

trsp_energy(trsp_fuel, engine_type, trsp_mode, reg, t) * trsp_conv(trsp_fuel, engine_type, trsp_mode) 	(31)

### 5.6.3 Non-PHEV, non-FCREEV vehicle capacity constraint

For conventional ICE, FC, hybrid and BEV drivetrains, the fuel use is proportional to the vehicle stock. The engine stock engines(trsp_fuel, non_phev, trsp_mode, reg, t) divided by the total fleet size num_veh(trsp_mode, reg, t) gives the fleet share assigned to a particular fuel-drivetrain combination. This share times the total mode demand gives the maximum service that combination can provide. The small term +0.0001 in the denominator avoids division by zero:

trsp_energy(trsp_fuel, non_phev, mode, reg, t) * trsp_conv(trsp_fuel, non_phev, mode) <=  engines(trsp_fuel, non_phev, mode, reg, t) / num_veh(mode, reg, t) * trsp_demand(mode, reg, t) 	(32)

### 5.6.4 PHEV liquid-fuel consumption

For plug-in hybrid vehicles, the liquid fuel use depends both on the vehicle stock share and the fraction of distance driven on fuel (1 – elec_frac_PHEV). The equation makes liquid fuel use exactly proportional to the ICE share of driving distance:

trsp_energy(road_fuel_liquid, 'PHEV', veh, reg, t)  =  engines(road_fuel_liquid, 'PHEV', veh, reg, t) / num_veh(veh, reg, t) * (1 - elec_frac_PHEV(veh)) * trsp_demand(veh, reg, t)/ (trsp_conv(road_fuel_liquid, 'PHEV', veh) + 0.0001) 	(33)

### 5.6.5 PHEV electricity-fuel split

The electricity and liquid fuel use in PHEVs must be consistent with the assumed electric driving fraction elec_frac_PHEV. This equation enforces that the ratio of electricity energy to liquid fuel energy (adjusted by their respective conversion factors) equals the ratio of electric to liquid driving shares:

SUM_{road_fuel_liquid} trsp_energy(road_fuel_liquid,'PHEV',veh,reg,t)*trsp_conv(road_fuel_liquid, 'PHEV', veh)/(1-elec_frac_PHEV)  =  trsp_energy('elec','PHEV',veh,reg,t) * trsp_conv('elec','PHEV',veh) / elec_frac_PHEV(veh) 	(34)

### 5.6.6 FCREEV vehicle constraints

The FC-REEV drivetrain operates analogously to the PHEV: FC fuels (H2, NH3, LCH4, LH2, MeOH, CH2) cover (1 – elec_frac_PHEV) of driving and electricity covers the remaining fraction. The vehicle_lim_FCREEV_Q equation sets FC fuel use in proportion to the engine stock and the non-electric driving share. The elec_frac_FCREEV_Q enforces the correct ratio of FC to electric energy use.

### 5.6.7 Electric vehicle fleet limit

The total stock of electrified drivetrains (BEV, PHEV, FCREEV) in any mode cannot exceed an exogenous maximum market share frac_engine(elec_veh, car_or_truck) of the total vehicle fleet. This represents infrastructure constraints, charging availability, and other adoption barriers:

SUM_{trsp_fuel} engines(trsp_fuel, elec_veh, car_or_truck, reg, t) <=  frac_engine(elec_veh, car_or_truck) * num_veh(car_or_truck, reg, t) 	(35)

### 5.6.8 Pilot fuel requirement

Hydrogen and ammonia ICE engines in shipping require a small amount of pilot fuel (petro or MeOH) to initiate ignition. The constraint ensures that at least 5% of the energy from NH3-, LH2-, LCH4- and CH2-ICE ships comes from pilot fuels:

SUM_{can_be_pilot} trsp_energy(can_be_pilot,'0',heavy_mode,reg,t) >=  0.05 * SUM_{need_pilot} trsp_energy(need_pilot,'0',heavy_mode,reg,t) 	(36)

## Investments and depreciation

### 5.7.1 Initial capital stock

In the initial year (2010), the capital stock equals the exogenously given initial capacity init_capital plus t_step times the new investment made in 2010. The factor t_step = 10 scales a one-year investment rate to a decade of capacity addition:

capital(en_in,en_out,type,reg, init_year)= init_capital(en_in,en_out,type,reg)  +  t_step * cap_invest(en_in,en_out,type,reg, init_year) 	(37)

### 5.7.2 Capital stock evolution

For all subsequent years, the capital stock in year t+1 equals the new investment in t+1 scaled by t_step, plus the surviving fraction of the existing stock. The surviving fraction is computed using the continuous depreciation formula exp(t_step * ln(1 – 1/life_plant)), which is equivalent to compounding a fixed annual retirement rate 1/life_plant over ten years:

capital(en_in,en_out,type,reg, t+1)= t_step * cap_invest(en_in,en_out,type,reg, t+1)  +  capital(en_in,en_out,type,reg, t) * exp( t_step * ln(1 - 1/life_plant(en_in,en_out,type)) ) 	(38)

### 5.7.3 Engine stock evolution

The vehicle engine stock evolves with the same depreciation formula. Rail modes are excluded (trains are not in the engine investment model):

engines(trsp_fuel,engine_type,mode,reg, t+1)  =  t_step * eng_invest(trsp_fuel,engine_type,mode,reg, t+1)+  engines(trsp_fuel,engine_type,mode,reg, t) * exp( t_step * ln(1 - 1/life_eng(trsp_fuel, engine_type, trsp_mode_no_train)) )	(39)

### 5.7.4 Infrastructure stock evolution

Fuel infrastructure (e.g. hydrogen filling stations, LNG bunkering) evolves analogously. New infrastructure investment is summed across all non-rail transport modes that share a given fuel type:

infra(trsp_fuel, reg, t+1)  =  t_step * SUM_{trsp_mode_no_train} infra_invest(trsp_fuel,mode,reg, t+1)+ infra(trsp_fuel, reg, t) * exp( t_step * ln(1 - 1/life_infra(trsp_fuel)) )	(40)

### 5.7.5 Capital limits on conversion

For all conversion routes except electricity and hydrogen (which use the sliced-sector equations), the annual energy output cannot exceed the installed capacity times load factor times seconds per year. The product capital * lf * Msec_per_year converts TW capacity into EJ/yr of maximum annual output:

en_conv(en_in,en_out,type,reg,t) * effic(en_in,en_out,type,reg,t)

<=  capital(en_in,en_out,type,reg,t) * lf(en_in,type,en_out) * Msec_per_year	(41)

### 5.7.6 Infrastructure capacity limit

The total energy delivered through transport fuel infrastructure (summed over all plant types) cannot exceed the infrastructure capacity times its load factor:

SUM_type en_conv(trsp_fuel,'trsp',type,reg,t) * effic(trsp_fuel,'trsp',type,reg,t) <=  infra(trsp_fuel, reg, t) * lf_infra(trsp_fuel) * Msec_per_year	(42)

## Emission calculations

### 5.8.1 Upstream emissions

Upstream emissions cover all stationary (non-transport) conversion activities. For each non-transport conversion, the emission equals the difference between the emission factor of the input fuel and the carbon embedded in the output carrier (which would count as an emission if later combusted). Permanently stored carbon is subtracted. Two additional terms account for fugitive methane: well-to-tank leakage from natural gas processing (WTT_CH4) and distribution leakage from liquefied methane for transport (Distrib_LCH4):

emiss_upstream(reg,t)  =  SUM_{en_out [not 'trsp']} SUM_{en_in,type}  en_conv(en_in,en_out,type,reg,t) * (emis_fact(en_in) - emis_fact(en_out)*effic(en_in,en_out,type,reg,t)) -  C_strg_tot(reg,t)+  SUM_{natural_gas,type} en_conv(natural_gas,'CH4',type,reg,t) * WTT_CH4+  SUM_(en_conv(LCH4/CH4,'trsp',type,reg,t) * Distrib_LCH4	(43)

### 5.8.2 Shipping emissions by vessel type

For each vessel category in heavy_mode, TTW CO₂ emissions are computed from the fuel energy multiplied by the fuel-specific TTW emission factor emis_fact2. This includes all fuel-drivetrain combinations for that vessel type:

ship_emis2(heavy_mode, reg, t)  =  SUM_{trsp_fuel, engine_type}  trsp_energy(trsp_fuel, engine_type, heavy_mode, reg, t)* emis_fact2(trsp_fuel)	(44)

### 5.8.3 Total ship emissions

Total regional shipping emissions aggregate over vessel types. Two additional emission terms are added: methane slip from LCH4-fuelled vessels (proportional to the fuel used, with slip rates that differ between large 2-stroke and smaller 4-stroke engines via extra_emis_CH4(heavy_mode)), and N₂O from ammonia ICE combustion:

ship_emis(reg,t)  =  SUM_{heavy_mode} ship_emis2(heavy_mode,reg,t)	(45)

+  SUM_{heavy_mode} trsp_energy('LCH4','0',heavy_mode,reg,t) * extra_emis_CH4(heavy_mode)

+  SUM_{heavy_mode} trsp_energy('NH3','0',heavy_mode,reg,t) * TTW_N2O

### 5.8.4 Road and aviation transport emissions

Non-heavy-mode transport emissions sum over all engine types, fuels and non-shipping modes, using the same TTW emission factor emis_fact2:

transp_emis(reg,t)  =  SUM_{engine_type, trsp_fuel, trsp_mode [not heavy_mode]} trsp_energy(trsp_fuel,engine_type,trsp_mode,reg,t) * emis_fact2(trsp_fuel) 	(46)

### 5.8.5 Total regional CO₂ emissions

Total regional emissions are the sum of upstream, shipping, and road/aviation transport emissions. This quantity is used in the cumulative budget constraint and in the carbon-tax cost calculation:

C_emission(reg,t)  =  emiss_upstream(reg,t) + ship_emis(reg,t) + transp_emis(reg,t)	(47)

### 5.8.6 Global emissions

Global annual emissions are the sum over regions:

SUM_reg C_emission(reg,t)  =  Tot_emis(t)	(48)

### 5.8.7 Aggregated cumulative emissions

The cumulative CO₂ emitted over the full model horizon is:

agg_emis  =  SUM_{t, reg} t_step * C_emission(reg, t)	(49)

### 5.8.8 Carbon capture

Total captured carbon per region per period includes CCS on fossil fuels and process gases, BECCS (biomass, biogas, pellets with technology-specific capture efficiencies and CO₂ content), and direct air capture. The biomass CO₂ content is approximated at 110 gCO₂/MJ for solid biomass, 50 gCO₂/MJ for biogas, and 80 gCO₂/MJ for pellets:

C_capt_tot(reg,t)  =  SUM_{ccs_fuel,en_out,C_capt} en_conv(ccs_fuel,en_out,C_capt,reg,t)* emis_fact(ccs_fuel) * capt_effic(ccs_fuel,C_capt,en_out)+  SUM_{en_out,C_capt} en_conv('bio',en_out,C_capt,reg,t) * 110 * capt_effic(ccs_fuel,C_capt,en_out)  +  SUM_{en_out,C_capt} en_conv('biogas',en_out,C_capt,reg,t) * 50 * capt_effic(ccs_fuel,C_capt,en_out)+  SUM_{en_out,C_capt} en_conv('pellets',en_out,C_capt,reg,t) * 80 * capt_effic(ccs_fuel,C_capt,en_out) +  C_capt_air(reg,t) 	(50)

### 5.8.9 Carbon balance and permanent storage

The carbon balance equation ensures that the carbon embedded in synthetic fuel production (MeOH and LCH4 from H2+CO2) equals the captured carbon minus the permanently stored carbon.

en_conv('H2','MeOH','0',reg,t)*emis_fact('MeOH')*effic(en_in, en_out, type,reg, t)  +  en_conv('H2','LCH4','0',reg,t)*emis_fact('LCH4')*effic(en_in, en_out, type,reg, t) =  C_capt_tot(reg,t)  -  C_strg_tot(reg,t)	 (51)

Cumulative permanent storage is bounded:

SUM_{t,reg} C_strg_tot(reg,t) * t_step  <=  C_strg_agg   	(52)

## Technology restrictions

### 5.9.1 Co-generation limit

Co-generation (CHP) is a valuable flexibility option, but cannot realistically supply the majority of heat demand, since not all heat users are co-located with electricity consumers. The heat from CHP plants that use cg_fuel fuels cannot exceed 60% (cogen_fr_h = 0.6) of the central heat demand:

SUM_{cg_fuel, cg_type} en_conv(cg_fuel,'elec',cg_type,reg,t) * heat_effic(cg_fuel,cg_type,'elec','central_heat')<=  cogen_fr_h * dem('central_heat', reg, t) 	(53)

### 5.9.2 Solar heat limits

Solar thermal heating is limited to a maximum fraction of the heat demand in each region (max_solar(heat, reg)), reflecting geographic and rooftop constraints. Separate limits apply to central heat and distributed or rural heat:

en_conv('solar', heat, '0', reg, t)  <=  dem(heat, reg, t) * max_solar(heat, reg)	(54)

### 5.9.3 Heat pump limits

The maximum fraction of heat demand supplied by electric heat pumps (max_pump(heat, reg)) reflects space and infrastructure constraints, and ranges from 20% for central heat to 60% for distributed heat:

en_conv('elec', heat, '0', reg, t)  <=  dem(heat, reg, t) * max_pump(heat, reg)	(55)

### 5.9.4 Biomass chip limit in solid heat

Wood chips cannot supply more than 67% of solid heat demand (max_chip = 0.67), reflecting limits on chip quality, moisture and logistics for industrial heat use:

SUM_type_en_conv('bio','solid_heat',type,reg,t)*effic('bio','solid_heat',type,reg,t)  <=  dem('solid_heat',reg,t)*max_chip	(56)

### 5.9.5 Feedstock limits

Each feedstock type (coal, petro, MeOH, CH4, etc.) cannot supply more than its maximum allowed fraction of feed-stock demand (max_feed(feed)). This reflects structural constraints on how each material is used in petrochemicals and fertiliser production:

SUM_type en_conv(feed,'feed-stock',type,reg,t)*effic(feed,'feed-stock',type,reg,t)  <=  dem('feed-stock',reg,t)*max_feed(feed)	(57)

## Expansion rate constraints

### 5.10.1 Primary supply growth

The supply of each primary fuel in t+1 cannot grow faster than supply_lim (10%/decade) relative to the previous period. The kick-start init_supply term (4 EJ/decade) ensures that new fuel categories can enter the model even if current production is very low:

supply_1(fuels, reg, t+1)  <=  supply_1(fuels, reg, t) * (1+supply_lim)^t_step  +  init_supply	(58)

### 5.10.2 Infrastructure growth

Infrastructure capacity for transport fuels cannot grow faster than 10%/decade (infra_g_lim), plus a kick-start init_infra. The same ratchet mechanism applies as for primary supply:

infra(trsp_fuel, reg, t+1)  <=  infra(trsp_fuel, reg, t) * (1+infra_g_lim)^t_step  +  init_infra	(59)

### 5.10.3 Engine fleet growth

The total stock of vehicles using each drivetrain type in each mode cannot grow faster than 20%/decade (eng_g_lim) relative to the current total fleet size. The constraint applies to the sum over all fuels for a given engine type, so that the total fleet share of (e.g.) BEVs cannot increase too rapidly, even if individual fuel-BEV combinations change:

SUM_trsp_fuel engines(trsp_fuel, engine_type, mode, reg, t+1)

<=  SUM_trsp_fuel engines(trsp_fuel, engine_type, mode, reg, t) * (1+eng_g_lim)^t_step  +  init_eng	(60)

### 5.10.4 Investment growth in conversion plants

New investment in conversion plants cannot increase faster than 25%/decade relative to the previous decade's investment level. This limits the speed of industrial build-out and reflects supply-chain and engineering workforce constraints:

cap_invest(en_in,en_out,type,reg, t+1) <=  cap_invest(en_in,en_out,type,reg, t) * (1+cap_g_lim)^t_step  +  init_plant	(61)

### 5.10.5 Coal phase-out protection

In the 2010 to 2020 transition, coal energy production in each sector is not allowed to fall by more than 10% per decade (i.e. must remain at least 90% of the 2010 level). This prevents the optimiser from instantaneously eliminating a large existing stock:

energy_prod2('coal', sector1, type, reg, 2020)  >=  energy_prod2('coal', sector1, type, reg, 2010) * 0.9	(62)

### 5.10.6 Biomass phase-out protection

After 2050, biomass use in each sector cannot increase faster than 20%/decade.

energy_prod2('bio', en_out, type, reg, t+1)  <=  1.2 * energy_prod2('bio', en_out, type, reg, t)  [for t >= 2050]	(63)

### 5.10.7 DAC growth limit

Direct air capture deployment follows the same investment growth constraint as conversion plants. This prevents instantaneous large-scale DAC rollout:

C_capt_air(reg, t+1)  <=  C_capt_air(reg, t) * (1+cap_g_lim)^t_step  +  init_plant	(64)

## Cost accounting

### 5.11.1 Primary energy extraction and import cost

The fuel cost for each region includes: (i) extraction cost from primary fuels at the regional supply price; (ii) import_cost of shipping capital costs per unit of imported energy carrier; and (iii) a distance-dependent transport cost imp_cost_lin per km of import route, multiplied by the bilateral distance table:

cost_fuel(reg, t)  =  SUM_fuels supply_1(fuels,reg,t) * price(fuels,reg)  +  SUM_{second_in} import(second_in,reg,t) * import_cost(second_in)+  SUM_{reg_exp, second_in} import_from(second_in,reg,reg_exp,t)* distance(reg,reg_exp) * imp_cost_lin(second_in)	(65)

### 5.11.2 Shipping levy cost

If the shipping levy policy is active, a levy (in kUSD/tCO₂) is applied to global shipping emissions. The revenue is collected at global level and not recycled to regions in the base levy scenario:

ship_levy_cost(t)  =  SUM_reg ship_emis(reg,t) * ship_levy(t)	(66)

### 5.11.3 Capital investment cost

The capital cost for each region is the sum of: (i) investment in conversion plants times the plant-specific investment cost; (ii) investment in vehicle engines times the vehicle investment cost (which includes the base vehicle chassis cost vehicle_cost); and (iii) investment in fuel infrastructure times the infrastructure cost:

cost_cap(reg,t)  =  SUM_{en_in,en_out,type}cap_invest(en_in,en_out,type,reg,t) * cost_inv(en_in,en_out,type,t)+  SUM_{trsp_fuel,engine_type,mode_no_train} eng_invest(trsp_fuel, engine_type, trsp_mode_no_train,reg, t) * cost_eng(trsp_fuel,engine_type,mode,t)  +  SUM_{trsp_fuel,mode_no_train} infra_invest(trsp_fuel,trsp_mode_no_train, reg, t) * cost_infra_mod(trsp_fuel,mode)	(67)

### 5.11.4 O&M cost

Operation and maintenance costs are calculated as a fraction OM_cost_fr of the annualised capital cost, times the annual energy throughput expressed in capacity units (en_conv * effic / (Msec_per_year * lf)). The OM_cost_fr is typically 4%, with lower values (1.5%) for nuclear and storage:

OM_cost(t)  =  SUM_{en_in,en_out,type,reg} OM_cost_fr(en_in,en_out) * cost_inv(en_in,en_out,type,t)  * en_conv(en_in,en_out,type,reg,t) * effic(...) / Msec_per_year / (lf(en_in,type,en_out)+0.000001)	(68)

### 5.11.5 Distribution cost

Distribution costs cover the cost of routing heat and fuels through networks. Two components: (i) a general infrastructure cost infra_cost(en_in, type, en_out) per unit of energy converted, applicable to technologies that require distribution infrastructure; and (ii) a heating distribution cost district_cost per EJ of co-generation heat delivered:

dist_cost(t)  =  SUM_{en_in,type,en_out,reg} en_conv(en_in,en_out,type,reg,t)*infra_cost(en_in,type,en_out)

+  SUM_reg supply_sec('central_heat',reg,t) * district_cost	(69)

### 5.11.6 Carbon storage cost

The total cost of capturing, transporting and permanently storing CO₂ is the amount permanently stored multiplied by the unit cost cost_strg (10 USD/tCO₂ in the base case, or equivalently 0.010 GUSD/MtCO₂):

cost_C_strg(t)  =  SUM_reg C_strg_tot(reg,t) * cost_strg	(70)

### 5.11.7 Direct air capture cost

The cost of direct air capture is the total CO₂ removed multiplied by the unit cost CO2aircapt_cost (0.5 GUSD/MtCO₂, equivalent to 500 USD/tCO₂):

cost_capt_air(t)  =  SUM_reg C_capt_air(reg,t) * CO2aircapt_cost	(71)

### 5.11.8 Carbon tax (optional)

If a carbon tax is active (C_tax(reg,t) > 0), the tax cost is the product of regional emissions and the tax rate. In the base case C_tax = 0 and the cumulative budget constraint is used instead:

tax(reg,t)  =  C_emission(reg,t) * C_tax(reg,t)	(72)

### 5.11.9 Total annual cost

The total annual energy-system cost in period t is the sum of all cost components. All cost components are summed earlier stage other that cost_fuel, cost_cap,and tax.

annual_cost(t)  =  SUM_reg cost_fuel(reg,t) + SUM_reg cost_cap(reg,t) + OM_cost(t) +  dist_cost(t) + cost_C_strg(t) cost_capt_air(t) +  SUM_reg tax(reg,t) + ship_levy_cost(t)	(73)

### 5.11.10 Objective function: total discounted cost

The objective function is the net present value of all annual costs, discounted at rate r = 5%/year. Each decade's cost is divided by the discount factor (1+r)^{t_step*(ord(t)-1)}, where ord(t)=1 for 2010, ord(t)=2 for 2020, etc.:

tot_cost  =  SUM_t  t_step * annual_cost(t) / (1+r)^{ t_step * (ord(t)-1) }	(74)

The model minimises tot_cost subject to all constraints above.

# Constraints and Scenario Settings

The file constraints.gms contains all fixed bounds, initial conditions, and scenario-specific constraints applied after the model equations are defined.

## CO₂ cumulative budget

The principal climate policy constraint is a cumulative CO₂ cap over 2020–2110 (CO2_cumulative_cap_Q). The value Cum_CO2_emis is set in constraints.gms. The values are set based on the RCP scenarios that is used for the analysis.

## Technology availability

The following constraints in constraints.gms enforce realistic deployment timelines:

| Constraint | Description |
| --- | --- |
| C_capt_air.fx = 0 for year < 2040 | DAC not available before 2040 |
| capital.fx(ccs_fuel,...) = 0 for year < 2030 | CCS not available before 2030 |
| cap_invest.up(nuclear,...) = 0.0001 for 2020, 2030 | Maximum ~10 new nuclear reactors per region in early periods |
| cap_invest.fx(FBF.elec, MOX.elec) = 0 | Fast-breeder and MOX reactors disabled in base case |
| engines.fx('NH3',...,p_ferry) = 0 | Ammonia excluded from passenger ferries |
| engines.fx('LH2',...,p_air_long/f_air) = 0 | Liquid H₂ excluded from long-haul and freight aviation |
| engines.fx(BEV/PHEV, f_HDT_*) = 0 for year < 2040 | BEV/PHEV heavy-duty trucks not before 2040 |
| engines.fx('elec','0',mode) = 0 | Pure electric ICE excluded (only BEV/PHEV allowed) |
| export.fx('elec'/'H2'/'CH2') = 0 | Electricity and gaseous hydrogen not traded between regions |
| EN_CONV.lo('CH4','elec','0',reg,t) for year<=2040 | Floor on gas-fired peaking plant to ensure grid stability |
| EN_CONV.lo('bio','solid_heat','0',reg,t) = 0.2*dem | Minimum 20% of solid heat from biomass (e.g. pulp/paper) |

## Calibration constraints

To ensure the model matches real-world 2020 data:

- en_conv.fx('wind','elec','0',reg,'2020') and en_conv.fx('solar','elec','0',reg,'2020') are fixed to BP Statistics 2020 values (wind_2020, solar_pv_2020 parameters divided by efficiency).

- en_conv.lo('EU4','elec','0',reg,'2020') sets a lower bound on nuclear based on nuclear_2020.

- en_conv.up('hydro','elec','0',reg,'2020') caps hydro at BP 2020 data (hydro_2020).

- en_conv.lo('wind/solar','elec','0',reg,'2030') imposes lower bounds from BP Statistics 2022 to anchor the 2030 decade.

- EN_CONV.fx('bio','dist_heat','0',reg,t) fixes traditional biomass use from the min_bio table, declining slowly from 2010 levels.

- C_emission.up(reg,'2010') caps 2010 emissions at 1.05 × historical value (emissions_2010).

- Primary fossil supply in 2020 is bounded using primary_cal_2020.

# Solver Settings and Implementation

GET 12 is solved as a linear programme (LP) using the CPLEX solver via GAMS. The solver options are specified through an option file (GET_12.optfile = 1). Key settings:

| Setting | Value | Description |
| --- | --- | --- |
| Solver | CPLEX | Industrial-grade LP solver |
| Iteration limit | 999,999,999 | Effectively unlimited |
| Resource (time) limit | 999,999,999 s | Effectively unlimited |
| LIMROW / LIMCOL | 0 | Suppress equation/variable listing in log |
| solprint | off | Suppress solution report in log |
| sysout | off | Suppress system output |
| solveopt | replace | Overwrite existing solution file |

The model is solved with: solve GET_12 using LP minimizing tot_cost.